Learn-/Training Document

Siemens Automation Cooperates with Education (SCE) | From Version V14 SP1

siemens.com/sce

TIA Portal Module 031-200

Basics of FB Programming with SIMATIC S7-1200

Matching SCE Trainer Packages for these Learn-/Training Document

- SIMATIC S7-1200 AC/DC/RELAY (set of 6) "TIA Portal"
Order no.: 6ES7214-1BE30-4AB3

- SIMATIC S7-1200 DC/DC/DC (set of 6) "TIA Portal"
Order no.: 6ES7214-1AE30-4AB3

- Upgrade SIMATIC STEP 7 BASIC V14 SP1 (for S7-1200) (set of 6) "TIA Portal"
Order no.: 6ES7822-0AA04-4YE5

Note that these trainer packages are replaced with successor packages when necessary.

An overview of the currently available SCE packages is available at: siemens.com/sce/tp

Continued training

For regional Siemens SCE continued training, get in touch with your regional SCE contact siemens.com/sce/contact

Additional information regarding SCE

siemens.com/sce

Information regarding use

The SCE Learn-/Training Document for the integrated automation solution Totally Integrated Automation (TIA) was prepared for the program "Siemens Automation Cooperates with Education (SCE)" specifically for training purposes for public educational facilities and R&D institutions. Siemens AG does not guarantee the contents.

This document is to be used only for initial training on Siemens products/systems, which means it can be copied in whole or part and given to those being trained for use within the scope of their training. Circulation or copying this Learn-/Training Document and sharing its content is permitted within public training and advanced training facilities for training purposes.

Exceptions require written consent from the Siemens AG contact person: Roland Scheuerer roland.scheuerer@siemens.com.

Offenders will be held liable. All rights including translation are reserved, particularly if a patent is granted or a utility model or design is registered.

Use for industrial customer courses is explicitly not permitted. We do not consent to commercial use of the Learn-/Training Document.

We wish to thank the TU Dresden, particularly Prof. Dr.-Ing. Leon Urbas and the Michael Dziallas Engineering Corporation and all other involved persons for their support during the preparation of this Learn-/Training Document.

Table of contents

1	Goal	5

2	Prerequisite	5

3	Required hardware and software	6

4	Theory	7

4.1	Operating system and application program	7

4.2	Organization blocks	8

4.3	Process image and cyclic program processing	9

4.4	Functions	11

4.5	Function blocks and instance data blocks	12

4.6	Global data blocks	13

4.7	Library-compatible logic blocks	14

4.8	Programming languages	15

5	Task	16

6	Planning	16

6.1	EMERGENCY STOP	16

6.2	Automatic mode - Conveyor motor	16

6.3	Technology diagram	17

6.4	Reference list	18

7	Structured step-by-step instructions	19

7.1	Retrieve an existing project	19

7.2	Create a new tag table	20

7.3	Create new tags within a tag table	22

7.4	Import "Tag_table_sorting_station"	23

7.5	Create function block FB1 "MOTOR_AUTO" for the conveyor motor in automatic mode	27

7.6	Define the interface of FB1 "MOTOR_AUTO"	29

7.7	Program FB1: MOTOR_AUTO	32

7.8	Program the organization block OB1 – Control conveyor tracking forwards in automatic mode	40

7.9	Result in the LAD (Ladder Logic) programming language	45

7.10	Save and compile the program	46

7.11	Download the program	47

7.12	Monitor program blocks	48

7.13	Archive the project	51

7.14	Checklist	52

8	Exercise	53

8.1	Task – Exercise	53

8.2	Technology diagram	53

8.3	Reference list	54

8.4	Planning	54

8.5	Checklist – Exercise	55

9	Additional information	56

Basics of FB Programming

# Goal

In this chapter, you will get to know the basic elements of a control program – the organization blocks (OBs), functions (FCs), function blocks (FBs) and data blocks (DBs). In addition, you will be introduced to library-compatible function und function block programming. You will get to know the Function Block Diagram (FBD) programming language and use it to program a function block (FB1) and an organization block (OB1).

The SIMATIC S7 controllers listed in Chapter 3 can be used.

# Prerequisite

This chapter builds on the hardware configuration of SIMATIC S7 CPU1214C. However, other hardware configurations that has digital input and output board be used. For this chapter, you can use the following project, for example:

SCE_EN_011_101_Hardware_Configuration_CPU1214C.zap14

# Required hardware and software

1	Engineering station: requirements include hardware and operating system 
(for additional information, see Readme on the TIA Portal Installation DVDs)

2	SIMATIC STEP 7 Basic software in TIA Portal – as of V14 SP1

3	SIMATIC S7-1200 controller, e.g. CPU 1214C DC/DC/DC with ANALOG OUTPUT SB1232 signal board, 1 AO – Firmware as of V4.2.1

Note: The digital inputs should be fed out to a control panel.

4	Ethernet connection between engineering station and controller

2 SIMATIC STEP 7 Basic (TIA Portal) as of V14 SP1

1 Engineering Station

4 Ethernet connection

3 SIMATIC S7-1200 controller

Control panel

# Theory

## Operating system and application program

Every controller (CPU) contains an operating system, which organizes all functions and sequences of the CPU that are not associated with a specific control task.

The tasks of the operating system include the following:

Performing a warm restart

Updating the process image of the inputs and output

Cyclically calling the user program

Detecting interrupts and calling interrupt OBs

Detecting and handling errors

Managing memory areas

The operating system is an integral component of the CPU and comes pre-installed.

The user program contains all functions that are necessary for executing your specific automation task. The tasks of the user program include the following:

Checking the basic requirements for a warm restart using startup OBs

Processing of process data, i.e. activation of output signals as a function of the input signal states

Reaction to interrupts and interrupt inputs

Error handling during normal program execution

## Organization blocks

Organization blocks (OBs) form the interface between the operating system of the controller (CPU) and the application program. They are called from the operating system and control the following operations:

Cyclic program processing (e.g. OB1)

Startup characteristics of the controller

Interrupt-driven program processing

Error handling

A project must have, at a minimum, an organization block for cyclic program processing. An OB is called by a start event as shown in Figure 1. In addition, the individual OBs have defined priorities so that, for example, an OB82 for error handling can interrupt the cyclic OB1.

Interruption

ON (Run)

Cyclic 
program processing

OB 1

Startup routine

OB 100 Warm restart
…

Interrupt-driven program processing

OB 40 …

Error handling

OB 80

OB 82

…

Interruption

Error

Interrupt

Cycle

Operating system

Figure 1: Start events in the operating system and OB calls

When a start event occurs, the following reactions are possible:

If an OB has been assigned to the event, this event triggers the execution of the assigned OB. If the priority of the assigned OB is greater than the priority of the OB that is currently being executed, it is executed immediately (interrupt). If not, the assigned OB waits until the higher-priority OB has been completely executed

If you have not assigned an OB to the event, the default system reaction is performed.

Table 1 shows examples for various start events for a SIMATIC S7-1200.  Also shown are the possible OB number(s) and the default system reactions that occur when the respective organization block (OB) is not present in the controller.

| Start event | Possible OB numbers | Default system reaction |
| --- | --- | --- |
| Startup | 100, 123 | Ignore |
| Cyclic program | 1, 123 | Ignore |
| Time-of-day interrupt | 10 to 11 | - |
| Update interrupt | 56 | Ignore |
| Scan cycle monitoring time exceeded once | 80 | Ignore |
| Scan cycle monitoring time exceeded twice | 80 | STOP |
| Diagnostic interrupt | 82 | Ignore |

Table 1: OB numbers for various start events

## Process image and cyclic program processing

When the cyclic user program addresses the inputs (I) and outputs (O), it does not query the signal states directly from the input/output modules. Instead, it accesses a memory area of the CPU. This memory area contains an image of the signal states and is called the process image.

The cyclic program processing sequence is as follows:

- At the start of the cyclic program, a query is sent to determine whether or not the individual inputs are energized. This status of the inputs is stored in the process image of the inputs (PII). In doing so, the information 1 or "High" is stored for energized inputs and the information 0 or "Low" for de-energized inputs.

- The CPU then executes the program stored in the cyclic organization block. For the required input information, the CPU accesses the previously read process image of the inputs (PII) and the results of logic operation (RLOs) are written to a so-called process image of the outputs (PIQ).

- At the end of the cycle, the process image of the outputs (PIQ) is transferred as the signal state to the output modules and these are energized or de-energized. The sequence then continues again with Item 1.

1. Save status of inputs in PII.

PII

PLC program in the program memory

1st instruction

2nd instruction

3rd instruction

4th instruction

...

Last instruction

2. Processing the program instruction-by-instruction with access to PII and PIQ

Local data

Bit memory

Data blocks

PIQ

3. Transfer status from the PIQ to the outputs.

Figure 2: Cyclic program processing

Note: The time the CPU needs for this sequence is called cycle time. This depends, in turn, on the number and type of instructions and the processor performance of the controller.

## Functions

Functions (FCs) are logic blocks without memory. They have no data memory in which values of block parameters can be stored. Therefore, all interface parameters must be connected when a function is called. To store data permanently, global data blocks must be created beforehand.

A function contains a program that is executed whenever the function is called from another logic block.

Functions can be used, for example, for the following purposes:

Math functions – that return a result dependent on input values.

Technological functions – such as individual controls with binary logic operations.

A function can also be called several times at different points within a program.

Organization block

Main [OB1]

Calls the MOTOR_MANUAL [FC1] function

MOTOR_MANUAL [FC1]

Contains a program for controlling a conveyor in manual mode, for example.

The function has no memory.

Figure 3: Function with call from organization block Main [OB1]

## Function blocks and instance data blocks

Function blocks are logic blocks that store their input, output and in-out tags as well as static tags permanently in instance data blocks, so that they are available after the block has been executed. For this reason, they are also referred to as blocks with "memory".

Function blocks can also operate with temporary tags. Temporary tags are not stored in the instance DB, however. Instead, they are only available for one cycle.

Function blocks are used for tasks that cannot be implemented with functions:

Whenever timers and counters are required in the blocks, or

When information must be stored in the program, such as preselection of the operating mode with a button.

Function blocks are always executed when called from another logic block. A function block can also be called several times at different points within a program. This facilitates the programming of frequently recurring complex functions.

A call of a function block is referred to as an instance. Each instance of a function block is assigned a memory area that contains the data that the function block uses. This memory is made available by data blocks created automatically by the software.

It is also possible to provide memory for multiple instances in one data block in the form of a multi-instance. The maximum size of instance data blocks varies depending on the CPU. The tags declared in the function block determine the structure of the instance data block.

Instance data block MOTOR_AUTO_DB1 [DB1] as memory

for the call

of function

block

MOTOR_AUTO	[FB1]

Organization block

Main [OB1]

Calls function block MOTOR_AUTO [FB1] together with its instance data block MOTOR_AUTO_DB1 [DB1]

Function block MOTOR_AUTO [FB1]

Contains a program for controlling a conveyor in automatic mode, for example.

The function block uses instance data block MOTOR_AUTO_DB1 [DB1] as memory in this call.

Figure 4: Function block and instance with call from organization block Main [OB1]

## Global data blocks

In contrast to logic blocks, data blocks contain no instructions. Rather, they serve as memory for user data.

Data blocks thus contain variable data that is used by the user program. You can define the structure of global data blocks as required.

Global data blocks store data that can be used by all other blocks (see Figure 5). Only the associated function block should access instance data blocks. The maximum size of data blocks varies depending on the CPU.

Global DB
(DB_Global)

Access only for function 
data block_12

Function 
block_12

Function_11

Function_10

Access for all blocks

Instance DB
(DB_Instance)

Figure 5: Difference between global DB and instance DB.

Application examples for global data blocks are:

Saving of information about a storage system. "Which product is located where?"

Saving of recipes for particular products.

## Library-compatible logic blocks

A user program can be created with linear or structured programming. Linear programming writes the entire user program in the cycle OB, but is only suitable for very simple programs for which other less expensive control systems, such as LOGO!, can now be used.

For more complex programs, structured programming is always recommended. Here, the overall automation task can be broken down into small sub-tasks in order to implement a solution for them in functions and function blocks.

In this case, library-compatible logic blocks should be created preferentially. This means that the input and output parameters of a function or function block are defined generally and only supplied with the current global tags (inputs/outputs) when the block is used.

Figure 6: Library-compatible function bock with call in OB1

## Programming languages

For SIMATIC S7-1200 controller, the available programming languages for programming functions and function blocks are Function Block Diagram (FBD), Ladder Logic (LAD) and Structured Control Language (SCL).

The Function Block Diagram (FBD) programming language will be presented in the following.

FBD is a graphical programming language. The representation is based on electronic switching systems. The program is mapped in networks. A network contains one or more logic operation paths. Binary and analog signals are linked together by boxes. The graphical logic symbols known from Boolean algebra are used to represent the binary logic.

You can use binary functions to query binary operands and to logically combine their signal states. The following instructions are examples of binary functions: "AND operation", "OR operation" and "EXCLUSIVE OR operation". These are shown in Figure 7.

Figure 7: Binary functions in FBD and associated logic table

You can thus use simple instructions, for example, to control binary outputs, evaluate edges and execute jump functions in the program.

Program elements such as IEC timers and IEC counters provide complex instructions.

The empty box serves as a placeholder in which you can select the required instruction.

Enable input EN (enable)/ Enable output ENO (enable output) mechanism:

An instruction without EN/ENO mechanism is executed independent of the signal state at the box inputs.

Instructions with EN/ENO mechanism are only executed if enable input "EN input has signal state "1". When the box is processed correctly, enable output "ENO" has signal state "1". If an error occurs during processing, the enable output "ENO" is reset. If enable input EN is not connected, the box is always executed.

# Task

The following functions of the sorting station process description will be planned, programmed and tested in this chapter:

Automatic mode - Conveyor motor

# Planning

The programming of all functions in OB1 is not recommended for reasons of clarity and reusability. The majority of the program code will therefore be moved into functions (FCs) and function blocks (FBs). The decision on which functions are to be moved to the FB and which is to run in OB 1 is planned below.

## EMERGENCY STOP

The EMERGENCY STOP does not require a separate function. Just like the operating mode, the current state of the EMERGENCY STOP relay can be used directly at the blocks.

## Automatic mode - Conveyor motor

Automatic mode of the conveyor motor is to be encapsulated in a function block (FB) "MOTOR_AUTO". On the one hand, this preserves the clarity of OB1. On the other hand, it enables reuse if another conveyor belt is added to the station. Table 2 lists the planned parameters.

| Input | Data type | Comment |
| --- | --- | --- |
| Automatic_mode_active | BOOL | Automatic mode activated |
| Start | BOOL | Pushbutton automatic start |
| Stop | BOOL | Pushbutton automatic stop |
| Enable_OK | BOOL | All enable conditions OK |
| Safety_shutoff_active | BOOL | Safety shutoff active e.g. emergency stop operated |
| Output |  |  |
| Conveyor_motor_automatic_mode | BOOL | Control of the conveyor motor in automatic mode |
| Static |  |  |
| Memory_automatic_start_stop | BOOL | Memory used for start/stop automatic mode |

Table 2: Parameters for FB "MOTOR_AUTO"

The Memory_automatic_start_stop is latched with Start but only if the reset conditions are not present.

The Memory_automatic_start_stop is reset if Stop is present or safety shutoff is active or automatic mode is not activated (manual mode).

The Conveyor_motor_automatic_mode output is controlled when Memory_automatic_start_stop is set and the enable conditions are met.

## Technology diagram

Here, you see the technology diagram for the task.

Figure 8: Technology diagram

Figure 9: Control panel

## Reference list

The following signals are needed as operands for this task.

| DI | Type | Identifier | Function | NC/NO |
| --- | --- | --- | --- | --- |
| I 0.0 | BOOL | -A1 | Return signal emergency stop ok | NC |
| I 0.1 | BOOL | -K0 | Main switch "ON" | NO |
| I 0.2 | BOOL | -S0 | Mode selector manual (0)/ automatic (1) | Manual = 0 Auto = 1 |
| I 0.3 | BOOL | -S1 | Pushbutton automatic start | NO |
| I 0.4 | BOOL | -S2 | Pushbutton automatic stop | NC |
| I 0.5 | BOOL | -B1 | Sensor cylinder M4 retracted | NO |

| DO | Type | Identifier | Function |  |
| --- | --- | --- | --- | --- |
| Q 0.0 | BOOL | -Q1 | Conveyor motor M1 forwards fixed speed |  |

Legend for reference list

| DO | Digital Output |
| --- | --- |
| AO | Analog Output |
| Q | Output |

| DI | Digital Input |
| --- | --- |
| AI | Analog Input |
| I | Input |
| NC | Normally Closed |
| NO | Normally Open |

# Structured step-by-step instructions

You can find instructions on how to carry out planning below. If you already have a good understanding of everything, it will be sufficient to focus on the numbered steps. Otherwise, simply follow the detailed steps in the instructions.

## Retrieve an existing project

Before we can start programming the function block (FB) "MOTOR_AUTO", we need a project with a hardware configuration (e.g. SCE_EN_011_101_Hardware_Configuration_S7-CPU1214C....zap). To retrieve an existing project that has been archived, you must select the relevant archive with  Project  Retrieve in the project view. Confirm your selection with Open ( Project  Retrieve  Select a .zap archive  Open).

The next step is to select the target directory where the retrieved project will be stored. Confirm your selection with "OK". ( Target directory  OK)

## Create a new tag table

In the project view, navigate to the  PLC tags of your controller and create a new tag table by double-clicking  Add new tag table.

Rename the tag table you just created as "Tag_table_sorting_station" ( right-click "Tag_table_1"  "Rename"  Tag_table_sorting_station).

Open this tag table with a double-click. ( Tag_table_sorting_station)

## Create new tags within a tag table

Add the name Q1 and confirm the entry with the Enter key. If you have not yet created additional tags, TIA Portal now automatically assigns data type "Bool" and address %I0.0 (I 0.0) ( <Add>  Q1  Enter).

Change the address to %Q0.0 (Q 0.0) by entering this directly or by clicking the drop-down arrow to open the Addressing menu. Change the operand identifier to Q and confirm with Enter or by clicking the check mark ( %I0.0  Operand identifier Q  )

Enter the "Conveyor motor M1 forwards fixed speed" comment for the tag.

Add a new Q2 tag in line 2. TIA Portal has automatically assigned the same data type as the one in line 1 and has incremented the address by 1 to %Q0.1 (Q0.1). Enter the comment "Conveyor motor M1 backwards fixed speed".

- ( <Add>  Q2  Enter  Comment  Conveyor motor M1 backwards fixed speed)

## Import "Tag_table_sorting_station"

To insert an existing symbol table, right-click on an empty field of the created "Tag_table_sorting_station". Select "Import file" in the shortcut menu.

- ( Right-click in an empty field of the tag table  Import file)

Select the desired symbol table (e.g. in .xlsx format) and confirm the selection with "Open".
( SCE_EN_020-100_Tag_table_sorting_station…  Open)

When the import is finished, you will see a confirmation window and have an opportunity to view the log file for the import. Click  OK.

You can see that some addresses have been highlighted in orange. These are duplicate addresses and the names of the associated tags have been numbered automatically to avoid confusion.

Delete the duplicate tags by selecting the lines and pressing the Del key on your keyboard or selecting "Delete" in the shortcut menu.

- ( Right-click on selected tags  Delete)

You now have a complete symbol table of the digital inputs and outputs in front of you. Save your project under the name 031-200_FB_Programming.

- ( Project  Save as ...  031-200_FB_Programming  Save)

## Create function block FB1 "MOTOR_AUTO" for the conveyor motor in automatic mode

In the PLC programming section of the portal view, click "Add new block" to create a new function block. ( PLC programming  Add new block  )

Rename your new block to: "MOTOR_AUTO", set the language to FBD and keep automatic assignment of the number. Select the "Add new and open" check box. You will thus be taken automatically to your created function block in the project view.Click "Add".

- ( Name: MOTOR_AUTOLanguage: FBD  Number: Automatic   Add new and open  Add)

## Define the interface of FB1 "MOTOR_AUTO"

If you selected "Add new and open", the project view opens with a window for creating the block you just added.

You can find the interface description of your function block in the upper section of your programming view.

A binary output signal is needed for controlling the conveyor motor. For this reason, we first create local output tag #Conveyor_motor_automatic_mode of the "Bool" type. Enter the comment "Control of the conveyor motor in automatic mode" for the parameter.
( Output Conveyor_motor_automatic_mode  Bool  Control of the conveyor motor in automatic mode)

Add parameter #Automatic_mode_active as the input interface under Input and confirm the entry with the Enter key or by exiting the entry field. Data type "Bool" is assigned automatically. This will be retained. Next, enter the associated comment "Automatic mode activated". 
( Input  Automatic_mode_active  Bool  Automatic mode activated)

Continue by adding parameters #Start, #Stop, #Enable_OK and #Safety_shutoff_active as additional binary input parameters and check their data types. Add descriptive comments.

The conveyor is started and stopped with pushbuttons. We therefore need a "Static" tag as a memory. Under Static, add tag #Memory_automatic_start_stop and confirm the entry with the Enter key or by exiting the entry field. Data type "Bool" is assigned automatically. This will be retained. Enter the associated comment "Memory used for start_stop automatic mode".
( Static  Memory_automatic_start_stop  Bool  Memory usedfor start/stop automatic mode)

For purposes of program documentation, assign the block title, a block comment and a helpful network title for Network 1.

- ( Block title: Motor control in automatic mode  Network 1: Memory_automatic_start_stop and control of the conveyor motor in automatic mode)

## Program FB1: MOTOR_AUTO

Below the interface description, you see a toolbar in the programming window with various logic functions and below that an area with networks. We have already specified the block title and the title for the first network there. Programming is performed within the networks using individual logic blocks. Distribution among multiple networks helps to preserve the clarity of the program. In the following, you will get to know the various ways you can insert logic blocks.

On the right side of your programming window is a list of instructions you can use in the program. Under  Basic instructions  Bit logic operations, find function  (Assignment) and use a drag & drop operation to move it to Network 1 (green line appears, mouse pointer with + symbol).

- ( Instructions  Basic instructions  Bit logic operations  )

Now use drag & drop to move your output parameter #Conveyor_motor_automatic_mode onto <??.?> above the block you just inserted. The best way to select a parameter in the interface description is by "grabbing" it at the blue symbol . (  Conveyor_motor_automatic_mode)

This determines that the #Conveyor_motor_automatic_mode parameter is written by this block. Still missing, however, are the input conditions so that this actually happens. An SR flip-flop and #Enable_OK parameter are logically combined with an AND logic operation at the input of the assignment block. To do this, first click the input of the block so that the input line has a blue background.

Click the  icon in your logic toolbar to insert an AND logic operation before your assignment block.

Use drag & drop to move input parameter #Enable_OK onto the second input of the & logic operation <??.?>. (  Enable_OK)

Use drag & drop to move the Set/reset flip-flop function  from the list of instructions under  Basic instructions  Bit logic operations onto the first input of the & operation .
( Instructions  Basic instructions  Bit logic operations     )

The SR flip-flop requires a memory tag. For this, use drag & drop to move static parameter #Memory_automatic_start_stop onto the <??.?> above the SR flip-flop.

- (  Memory_automatic_start_stop)

The #Memory_automatic_start_stop will be set with input tag #Start. Click twice on the S input of the SR flip-flop <??.?>  and enter "Start" in the field that appears in order to see a list of available tags starting with "Start".Click the #Start tag and apply with  Enter.
( SR flip-flop  <??.?> Start  #Start  Enter)

Note: When assigning tags in this way, there is a risk of a mix-up with the global tags from the tag table. The previously presented procedure using drag & drop from the interface description should therefore be used preferentially.

Multiple conditions are to be able to stop the conveyor. An OR block is therefore needed at the R1 input of the SR flip-flop. First, click the R1 input of the SR flip-flop so that the input line has a blue background.

Click the  icon in your logic toolbar to insert an OR logic operation.

The OR block has 2 inputs initially. In order to logically combine an additional input tag, click the yellow star  of your OR block.

Add input tags #Stop, #Safety_shutoff_active and #Automatic_mode_active to the 3 inputs of the OR block.

Negate the input connected to parameter #Automatic_mode_active by selecting it and clicking .

Do not forget to click  regularly. The finished function block "MOTOR_AUTO" [FB1] in FBD is shown below.

Under "General" in the properties of the block, you can change the "Language" to LAD (Ladder Logic) (Properties  General  Language: LAD)

The program has the following appearance in LAD.

## Program the organization block OB1 – Control conveyor tracking forwards in automatic mode

Before programming organization block "Main [OB1]", we switch the programming language to FBD (Function Block Diagram). To do so, first click on "Main [OB1]" in the "Program blocks" folder.

- ( CPU_1214C [CPU 1214C DC/DC/DC]  Program blocks  Main [OB1]  Switch programming language  FBD)

Open the "Main [OB1]" organization block with a double-click.

Assign Network 1 the name "Control conveyor tracking forwards in automatic mode"
( Network 1:... Control conveyor tracking forwards in automatic mode)

Use drag & drop to move your "MOTOR_AUTO [FB1]" function block onto the green line in Network 1.

The instance data block for this call of FB1 is created automatically. Assign a name and apply it with OK. ( MOTOR_AUTO_DB1  OK)

A block with the interface you defined, the instance data block and connections EN and ENO are inserted in Network 1.

To insert an AND before input parameter "Enable_OK", select this input and insert the AND by clicking the  icon in your logic toolbar ().

To connect the block to the global tags from "Tag_table_sorting_station", we have two options:

- Either select the "Tag_table_sorting_station" in the project tree and use drag & drop to move the desired global tag from the Details view to the interface of FC1
(  Tag_table_sorting_station  Details view  -S0  Automatic_mode_active)

Or, enter the starting letters (e.g. "-S") of the desired global tag for <??.?> and select the global input tag "-S0" (%I0.2) from the displayed list

- ( Automatic_mode_active  -S  -S0).

Insert the other input tags "-S1", "-S2", "-K0", "-B1" and "-A1" and then insert output tag 
"-Q1" (%Q0.0) at output "Conveyor_motor_automatic_mode".

Negate the querying of input tags "-S2" and "-A1" by selecting them 
and clicking  ( -S2    -A1  ).

## Result in the LAD (Ladder Logic) programming language

The result in the LAD (Ladder Logic) programming language has the following appearance.

## Save and compile the program

To save your project, select the  button in the menu. To compile all blocks, click the "Program blocks" folder and select the  icon for compiling in the menu 
(   Program blocks  ).

The "Info", "Compile" area shows which blocks were successfully compiled.

## Download the program

After successful compilation, the complete controller with the created program, as previously described in the modules for hardware configuration, can be downloaded ( ).

## Monitor program blocks

The desired block must be open for monitoring the downloaded program. The monitoring can now be activated/deactivated by clicking the   icon ( Main [OB1]  ).

Note: The monitoring here is signal-related and controller-dependent. The signal states at the terminals are indicated with TRUE or FALSE.

The "MOTOR_AUTO" [FB1] function block called in the "Main [OB1]" organization block can be selected directly for "Open and monitor" after right-clicking ( "MOTOR_AUTO" [FB1]  Open and monitor).

Note: The monitoring here is function-related and controller-independent. The actuation of sensors and the station status are shown here with TRUE or FALSE.

If a particular point of use of a "MOTOR_AUTO" [FB1] function block that is called multiple times is to be monitored, this can be performed using the  icon. There are two alternatives available for specifying the call environment: using the call environment or the instance data block (   Instance data block  MOTOR_AUTO_DB1 [DB1]  Call environment  Address: OB1  Details: Main NW1  OK).

## Archive the project

As the final step, we want to archive the complete project. Select the  "Archive ..." command in the  "Project" menu. Select a folder where you want to archive your project and save it with the file type "TIA Portal project archive". ( Project  Archive  TIA Portal project archive  031-200_FB Programming….  Save)

## Checklist

| No. | Description | Completed |
| --- | --- | --- |
| 1 | Compiling successful and without error message |  |
| 2 | Download successful and without error message |  |
| 3 | Switch on station (-K0 = 1) Cylinder retracted / Feedback activated (-B1 = 1) EMERGENCY OFF (-A1 = 1) not activated AUTOMATIC mode (-S0 = 1) Pushbutton automatic stop not actuated (-S2 = 1) Briefly press the automatic start pushbutton (-S1 = 1) Conveyor motor forwards fixed speed then switches on (-Q1 = 1) switches on and stays on. |  |
| 4 | Briefly press the automatic stop pushbutton (-S2 = 0) -Q1 = 0 |  |
| 5 | Activate EMERGENCY OFF (-A1 = 0) -Q1 = 0 |  |
| 6 | Manual mode (-S0 = 0) -Q1 = 0 |  |
| 7 | Switch off station (-K0 = 0) -Q1 = 0 |  |
| 8 | Cylinder not retracted (-B1 = 0) -Q1 = 0 |  |
| 9 | Project successfully archived |  |

# Exercise

## Task – Exercise

In this exercise, an energy saving function is to be added to the MOTOR_AUTO [FB1] function block. The expanded function block will be planned, programmed and tested:

To save energy, the conveyor should only run when a part is present.

The Conveyor_motor_automatic_mode output is therefore only activated when Memory_automatic_start_stop is set, the enable conditions are met and Memory_conveyor_start_stop is set.

The Memory_conveyor_start_stop is set when Sensor_chute_occupied signals a part and is reset when Sensor_end_of_conveyor produces a negative edge or safety shutoff is active or automatic mode is not activated (manual mode).

## Technology diagram

Here, you see the technology diagram for the task.

Figure 10: Technology diagram

Figure 11: Control panel

## Reference list

The following signals are needed as global operands for this task.

| DI | Type | Identifier | Function | NC/NO |
| --- | --- | --- | --- | --- |
| I 0.0 | BOOL | -A1 | Return signal emergency stop ok | NC |
| I 0.1 | BOOL | -K0 | Main switch "ON" | NO |
| I 0.2 | BOOL | -S0 | Mode selector manual (0)/ automatic (1) | Manual = 0 Auto = 1 |
| I 0.3 | BOOL | -S1 | Pushbutton automatic start | NO |
| I 0.4 | BOOL | -S2 | Pushbutton automatic stop | NC |
| I 0.5 | BOOL | -B1 | Sensor cylinder M4 retracted | NO |
| I 1.0 | BOOL | -B4 | Sensor at chute occupied | NO |
| I 1.3 | BOOL | -B7 | Sensor part at end of conveyor | NO |

| DO | Type | Identifier | Function |  |
| --- | --- | --- | --- | --- |
| Q 0.0 | BOOL | -Q1 | Conveyor motor M1 forwards fixed speed |  |

Legend for reference list

| DO | Digital Output |
| --- | --- |
| AO | Analog Output |
| Q | Output |

| DI | Digital Input |
| --- | --- |
| AI | Analog Input |
| I | Input |
| NC | Normally Closed |
| NO | Normally Open |

## Planning

Plan the implementation of the task on your own.

Note: Learn about the use of the negative edge in SIMATIC S7-1200 in the online help.

## Checklist – Exercise

| No. | Description | Completed |
| --- | --- | --- |
| 1 | Compiling successful and without error message |  |
| 2 | Download successful and without error message |  |
| 3 | Switch on station (-K0 = 1) Cylinder retracted / Feedback activated (-B1 = 1) EMERGENCY OFF (-A1 = 1) not activated AUTOMATIC mode (-S0 = 1) Pushbutton automatic stop not actuated (-S2 = 1) Briefly press the automatic start pushbutton (-S1 = 1) Sensor at chute activated (-B4 = 1) Conveyor motor forwards fixed speed then switches on (-Q1 = 1) switches on and stays on. |  |
| 4 | Sensor at end of conveyor activated (-B7 = 1) -Q1 = 0 |  |
| 5 | Briefly press the automatic stop pushbutton (-S2 = 0) -Q1 = 0 |  |
| 6 | Activate EMERGENCY OFF (-A1 = 0) -Q1 = 0 |  |
| 7 | Manual mode (-S0 = 0) -Q1 = 0 |  |
| 8 | Switch off station (-K0 = 0) -Q1 = 0 |  |
| 9 | Cylinder not retracted (-B1 = 0) -Q1 = 0 |  |
| 10 | Project successfully archived |  |

# Additional information

- More information for further practice and consolidation is available as orientation, for example: Getting Started, videos, tutorials, apps, manuals, programming guidelines and trial software / firmware, under the following link:

www.siemens.com/sce/s7-1200

Preview „Additional information“

Further Information

Siemens Automation Cooperates with Education
siemens.com/sce

SCE Learn-/Training Documents
siemens.com/sce/documents

SCE Trainer Packages
siemens.com/sce/tp

SCE Contact Partners 
siemens.com/sce/contact

Digital Enterprise
siemens.com/digital-enterprise

Industrie 4.0 
siemens.com/future-of-manufacturing

Totally Integrated Automation (TIA)
siemens.com/tia

TIA Portal
siemens.com/tia-portal

SIMATIC Controller
siemens.com/controller

SIMATIC Technical Documentation 
siemens.com/simatic-docu

Industry Online Support
support.industry.siemens.com

Product catalogue and online ordering system Industry Mall 
mall.industry.siemens.com

Siemens AG
Digital Factory 
P.O. Box 4848
90026 Nuremberg
Germany

Subject to change and errors
© Siemens AG 2018

siemens.com/sce