TIA Portal Modul 012-110

Specified Hardware Configuration

with SIMATIC S7-1500
CPU 1512SP F-1 PN

siemens.com/sce

Learn-/Training Document

Siemens Automation Cooperates with Education (SCE) | From Version V14 SP1

Matching SCE Trainer Packages for this Learn-/Training Document

SIMATIC Controllers

- SIMATIC CPU 1512SP F-1 PN with Software			
Order no.: 6ES7512-1SK00-4AB2

- SIMATIC ET 200SP - Analog Modules Expansion		
Order no.: 6ES7155-6AU00-0AB6

SIMATIC STEP 7 Software for Training

- SIMATIC STEP 7 Professional V14 SP1 - Single license

Order no.: 6ES7822-1AA04-4YA5

- SIMATIC STEP 7 Professional V14 SP1 - Classroom license (up to 6 users)

Order no.: 6ES7822-1BA04-4YA5

- SIMATIC STEP 7 Professional V14 SP1 - Upgrade license (up to 6 users)

Order no.: 6ES7822-1AA04-4YE5

- SIMATIC STEP 7 Professional V14 SP1 - Student license (up to 20 users)

Order no.: 6ES7822-1AC04-4YA5

Note that these trainer packages are replaced with successor packages when necessary.

An overview of the currently available SCE packages is available at: siemens.com/sce/tp

Continued training

For regional Siemens SCE continued training, get in touch with your regional SCE contact siemens.com/sce/contact

Additional information regarding SCE

siemens.com/sce


Information regarding use

The SCE Learn-/Training Document for the integrated automation solution Totally Integrated Automation (TIA) was prepared for the program "Siemens Automation Cooperates with Education (SCE)" specifically for training purposes for public educational facilities and R&D institutions. Siemens does not guarantee the contents.

This document is only to be used for initial training on Siemens products/systems. This means it can be copied in whole or in part and given to trainees/students for use within the scope of their training/course of study. Disseminating or duplicating this document and sharing its content is permitted within public training and advanced training facilities for training purposes or as part of a course of study.

Exceptions require written consent from Siemens. Send all related requests to 
scesupportfinder.i-ia@siemens.com.

Offenders will be held liable. All rights including translation are reserved, particularly if a patent is granted or a utility model or design is registered.

Use for industrial customer courses is explicitly not permitted. We do not consent to commercial use of the Learn-/Training Document.

We wish to thank the TU Dresden, particularly Prof. Dr.-Ing. Leon Urbas and the Michael Dziallas Engineering Corporation and all other involved persons for their support during the preparation of this Learn-/Training Document.

Table of contents

1	Objective	5

2	Requirement	5

3	Hardware and software required	5

4	Theory	7

4.1	Automation system SIMATIC S7-1500 with ET 200SP CPU	7

4.2	Installation and operation of the SIMATIC ET 200SP CPU	8

4.2.1	Range of modules	9

4.2.2	Example configuration	13

4.3	Operator controls and display elements of the CPU 1512SP  F-1 PN	14

4.3.1	Front view of the CPU 1512SP F-1 PN with BA 2xR bus adapter	14

4.3.2	Status and error displays	15

4.3.3	SIMATIC memory card	15

4.3.4	Mode switch	16

4.3.5	Memory areas of the CPU 1512SP F-1 PN and the SIMATIC memory card	17

4.4	Programming software STEP 7 Professional V14 (TIA Portal V14)	18

4.4.1	Project	19

4.4.2	Hardware configuration	19

4.4.3	Centralized and distributed automation structure	20

4.4.4	Planning the hardware	20

4.4.5	TIA Portal – Project view and portal view	21

4.4.6	Basic settings for the TIA Portal	23

4.4.7	Setting the IP address on the programming device	25

4.4.8	Setting the IP address in the CPU	28

4.4.9	Formatting a memory card in the CPU	31

4.4.10	Resetting the CPU to factory settings	32

4.4.11	Reading out the firmware version of the CPU 1512SP F-1 PN	33

5	Task	34

6	Planning	34

7	Structured step-by-step instructions	35

7.1	Creating a new project	35

7.2	Inserting the CPU 1512SP F-1 PN	36

7.3	Configuration of the Ethernet interface of the CPU 1512SP F-1 PN	40

7.4	Configuration of fail-safe operation of the CPU 1512SP F-1 PN	41

7.5	Configuration of access level for the CPU 1512SP F-1 PN	42

7.6	Inserting the digital input modules DI 8x24VDC HF	43

7.7	Inserting the digital output modules DQ 8xDC24V / 0.5A HF	45

7.8	Changing components in the hardware configuration	46

7.9	Inserting the server module	47

7.10	Configuration of address ranges for DI/DO: 0…1	48

7.11	Configuration of the potential groups of the BaseUnits	49

7.12	Saving and compiling the hardware configuration	51

7.13	Downloading the hardware configuration to the device	53

7.14	Downloading the hardware configuration to the PLCSIM simulation (optional)	58

7.15	Archiving the project	66

7.16	Checklist – step-by-step instructions	67

8	Exercise	68

8.1	Task description – Exercise	68

8.2	Planning	68

8.3	Checklist – Exercise	69

9	Additional information	70

Specific hardware configuration –

SIMATIC S7-1500 CPU 1512SP F-1 PN

# Objective

In this section, you will start by learning how to create a project. In a portion of a task, you will also be shown how to recognize previously installed hardware and add it to a project using the TIA Portal. You will then be shown how to configure the hardware.

The SIMATIC S7 controllers listed in chapter 3 can be used.

# Requirement

Nothing is required from other chapters to successfully complete this chapter.

# Hardware and software required

1	Engineering Station: hardware and operating system (for additional information, see Readme file on the TIA Portal Installation DVDs)

2	SIMATIC STEP 7 Professional software in the TIA Portal V14 or higher

3	SIMATIC S7-1500 controller with ET 200SP CPU Firmware V2.0 or higher with memory card and at least 16 DI/16 DO as well as 2 AI/1 AO

Example configuration
1x controller CPU 1512SP F-1 PN with bus adapter BA 2xRJ45
2x IO module 8x digital input DI 8x24VDC HF
2x IO module 8x digital output DQ 8x24VDC/0.5A HF
2x IO module 2x analog input AI 2xU/I 2.4-wire HS
1x IO module 2x analog output AQ 2xU/I HS
1x server module

4	Ethernet connection between the engineering station and controller

2 STEP 7 Professional (TIA Portal) V14 or higher

1 Engineering station

4 Ethernet connection

3 SIMATIC S7-1500 controller with ET 200SP CPU

# Theory

## Automation system SIMATIC S7-1500 with ET 200SP CPU

The SIMATIC S7-1500 automation system with the ET 200SP CPU is a modular control system for the medium-level and high-level performance range. A comprehensive range of modules is available for optimum adaptation to the automation task.

The CPUs of the ET 200SP CPU series represent a further development of the ET 200S CPU series and have the following new performance features:

- Increased system performance

- Integrated Motion Control functionality

- PROFINET IO IRT

- OPC UA

- STEP 7 language innovations while retaining proven functions

## Installation and operation of the SIMATIC ET 200SP CPU

SIMATIC ET 200SP is mounted on a standard mounting rail ⑦ and consists of the CPU/interface module ① with bus adapter ⑥, up to 64 input and output modules for digital and analog signals ④ plugged onto BaseUnits ②, ③ and a terminating server module ⑤. Communications processors and function modules for special tasks, such as PROFIBUS communication, IO-Link, PROFIenergy and stepper motor control, are also used when necessary.

①

④

②

As a programmable logic controller (PLC), SIMATIC ET 200SP CPU monitors and controls a machine or process with the S7 program. The IO modules are queried and addressed in the S7 program using input addresses (%I) and output addresses (%Q).

The system is programmed with the STEP 7 Professional software in the TIA Portal.

### Range of modules

The SIMATIC S7-1500 controller of the ET 200SP CPU series is a modular automation system with the following range of modules:

Central processing units (CPUs) with pluggable bus adapters

The CPUs, which are available for different performance levels, run the user program. In addition, the integrated system power supply provides power to the other modules over the backplane bus.

Bus adapters enable users to use the connection system of their choice.

Additional properties and functions of the CPU:

Communication via Ethernet

Communication via PROFIBUS/PROFINET

HMI communication on operator control and monitoring devices

Web server

Integrated technology functions (e.g.: PID controller, Motion Control, etc.)

System diagnostics

Trace functionality

Integrated security (e.g.: know-how, copy, access, data integrity protection)

BaseUnits

As universal base modules for electrical and mechanical connection of IO-modules.

They are available in a light variant BU..D, which opens a new potential group over the backplane bus for the power supply, and a dark variant BU..B, which continues the potential group. Thus, at least one BaseUnit BU..D must be used in order to supply at least one potential group with voltage. The IO-modules are plugged onto the BaseUnits.

IO modules

For digital input (DI) / digital output (DQ) / analog input (AI) / analog output (AQ).

They are available in variants for 24 V DC ①, 400 V AC ② and F-modules (fail-safe) ③.

③

②

①

Communication modules (CM)

For a point-to-point connection (PtP) ① or for connection to the communication systems IO-Link ②, AS-i ③, etc.

③

②

①

Technology modules (TM)

Modules for detecting high-frequency pulses, for weighing and for positioning, etc.

Server module

As termination of the ET 200SP system installation.

It can be used as a holder for 3 standby fuses. It serves as a terminating resistor for the backplane bus and is thus required.

SIMATIC memory card

Up to a maximum of 32 GB for storing program data and for easier replacement of CPUs during maintenance.

### Example configuration

The following configuration of a SIMATIC S7-1500 controller of the ET 200SP CPU series is used for the program example in this document.

⑥

⑤

④

⑥

⑤

④

③

②

①

① Central processing unit CPU 1512SP F-1 PN with bus adapter BA 2xRJ45

② 2x IO module 8x digital input DI 8x24VDC HF (2x)

③ 2x IO module 8x digital output DQ 8x24VDC/0.5A HF (2x)

④ 2x IO module 2x analog input AI 2xU/I 2.4-wire HS (2x)

⑤ IO module 2x analog output AQ 2xU/I HS (1x)

⑥ Server module

## Operator controls and display elements of the CPU 1512SP 
F-1 PN

The following figure shows the operator controls and display elements of a CPU 1512SP F-1 PN and a BA 2xRJ45 bus adapter. For other CPUs, the number of elements and their arrangement differ from this figure.

### Front view of the CPU 1512SP F-1 PN with BA 2xR bus adapter

① Mounting rail release

② Labeling strips

③ LEDs for status and error displays

④ LED for display of supply voltage

⑤ Mode switch

⑥ Slot for the SIMATIC memory card

⑦ Connection for supply voltage (included in the product package)

⑧ Cable support and fixing means for Port P3 of the PROFINET interface

⑨ LEDs for status displays of the PROFINET interface for ports P1, P2 and P3

⑩ Port P3 of the PROFINET interface: RJ45 socket on the CPU

⑪ Single view of the bus adapter

⑫ Port P1 R of the PROFINET interface: RJ45 socket on bus adapter BA 2×RJ45

⑬ Port P2 R of the PROFINET interface: RJ45 socket on bus adapter BA 2×RJ45

### Status and error displays

The CPU 1512SP-1 PN and bus adapter BA 2xRJ45 are equipped with the following LEDs:

① RUN/STOP-LED (yellow/green LED)

② ERROR LED (red LED)

③ MAINT LED (yellow LED)

④ LINK RX/TX-LED for ports X1 P1 and X1 P2 (green LEDs on bus adapter)

⑤ POWER LED (green LED)

⑥ LINK RX/TX LED for port X1 P3 (green LED on CPU)

### SIMATIC memory card

A SIMATIC micro memory card (MMC) is used as a memory module for the CPUs. This is a pre-formatted memory card that is compatible with the Windows file system. It is available in different memory sizes and for the following purposes:

Portable data storage medium

Program card

Firmware update card

The MMC must be inserted for operation of the CPU because the CPUs have no integrated load memory. A commercially available SD card reader is needed for reading/writing of the SIMATIC memory card with the programming device or PC. Files can then be copied directly to the SIMATIC memory card with Windows Explorer.

Note:

It is recommended that the SIMATIC memory card only be inserted and removed when the CPU is in POWER OFF state.

### Mode switch

You can use the mode switch to set the current operating mode of the CPU. The mode switch is designed to toggle 3 switch positions.

| Position | Meaning | Explanation |
| --- | --- | --- |
| RUN | RUN mode | The CPU runs the user program. |
| STOP | STOP mode | The CPU does not run the user program. |
| MRES | Memory reset | Position for memory reset of the CPU |

You can also switch the operating mode (STOP or RUN) with the button under Online & diagnostics on the CPU operator panel of the STEP 7 Professional V14 software.

In addition, the operator panel contains an MRES button for resetting the memory and shows the status LEDs of the CPU.

### Memory areas of the CPU 1512SP F-1 PN and the SIMATIC memory card

The following figure shows the memory areas of the CPU and the load memory on the

SIMATIC memory card.

In addition to the load memory, it is possible to load additional data onto the SIMATIC memory card with Windows Explorer. Recipes, data logs, project backups and other documentation for the program can be loaded there, for example.

Load memory

(on SIMATIC memory card)

- Code blocks (FC, FB, OB)

- Data blocks (DB)

- Hardware configuration

- Technology objects

Other memory areas

- Bit memory, timers, counters

- Temporary local data

- Process images (I/O)

Retentive memory

Parts of:

- Global data blocks

- Instance data blocks

- Technology objects

- Bit memory, timers, counters

Data work memory

- Global data blocks

- Instance data blocks

- Technology objects

Code work memory

- Code blocks (FC, FB, OB)

Load memory

Load memory is non-volatile memory for code blocks, data blocks, technology objects and the hardware configuration. When these objects are downloaded to the CPU, they are first stored in the load memory. This memory is located on the SIMATIC memory card.

Work memory

Work memory is volatile memory that contains the code and data blocks. It is integrated in the CPU and cannot be expanded. The work memory is divided into two areas in S7-1500 CPUs:

Code work memory: 
Code work memory contains parts of the program code relevant for execution.

Data work memory 
Data work memory contains parts of the data blocks and technology objects relevant for execution.

During the operating mode transition from POWER ON to Start and from STOP to Start, tags of global data blocks, instance data blocks and technology objects are initialized with their start values. Retentive tags keep their current values saved in the retentive memory.

Retentive memory

Retentive memory is non-volatile memory for backing up certain data in case of power failure. The tags and operand areas defined as retentive are saved in retentive memory. This data is retained for the duration of a shutdown or power failure.

All other program tags are set to their start values with an operating mode transition from POWER ON to Start and from STOP to Start.

The content of the retentive memory is deleted by the following actions:

Memory reset

Reset to factory settings

Note:

Certain tags of technology objects are also saved in retentive memory. These are not deleted with a memory reset.

## Programming software STEP 7 Professional V14 (TIA Portal V14)

The STEP 7 Professional V14 (TIA Portal V14) software is the programming tool for the following automation systems:

SIMATIC S7-1500

SIMATIC S7-1200

SIMATIC S7-300

SIMATIC S7-400

SIMATIC WinAC

With STEP 7 Professional V14, the following system automation functions can be used:

Configuration and parameter assignment of the hardware

Specifying the communication

Programming

Testing, commissioning and service with operational/diagnostic functions

Documentation

Creation of visualizations for SIMATIC Basic Panels with integrated WinCC Basic.

Visualization solutions for PCs and other panels can also be created with other WinCC packages.

All functions are supported by an extensive online help.

### Project

You create a project in the TIA Portal for performing an automation and visualization task. A project in the TIA Portal contains the configuration data for the installation and networking of devices as well as the programs and the configuration of the visualization.

### Hardware configuration

The hardware configuration contains the configuration of the devices, consisting of the automation system hardware, the intelligent field devices and the hardware for visualization. The configuration of the networks specifies the communication between the various hardware components. Individual hardware components are inserted from catalogs into the hardware configuration.

The automation system hardware is composed of controllers (CPUs), signal modules for input and output signals (SMs) and communication and interface modules (CP, IM). System power supply and load power supply modules (PS, PM) are also available for supplying energy to the modules.

The signal modules and the intelligent field devices connect the input and output data process that is to be automated and visualized to the automation system.

Distributed structure

Actuators/sensors

Centralized structure

Visualization

Figure 1: Example of hardware configuration with centralized and distributed structures

The hardware configuration enables automation and visualization solutions to be downloaded to the automation system and the controller to have access to the connected signal modules.

### Centralized and distributed automation structure

Figure 1 shows an automation structure containing both centralized and distributed structures.

In centralized structures, the input and output signals of the process are transmitted over conventional wiring to the signal modules, which are connected directly to the controller. Conventional wiring means that sensors and actuators are connected using 2-wire or 4-wire cables.

These days, the use of distributed structures is predominant. In this case, the sensors and actuators only have to be conventionally wired up to the signal modules of the field devices. The signal is transmitted from the field devices to the controller via an industrial communication system.

Both classic fieldbuses such as PROFIBUS, Modbus and Foundation Fieldbus and Ethernet-based communication systems such as PROFINET are used for industrial communication.

The communication system can also be used to connect intelligent field devices on which independent programs run. These programs can also be created with the TIA Portal.

### Planning the hardware

Before you can configure the hardware, you must plan it. In general, you start by selecting the required controllers and the number needed. Next, you select the communication modules and signal modules. The signal modules are selected based on the number and type of inputs and outputs needed. Finally, an adequate power supply must be selected for each controller or field device.

The functionality needed and the environmental conditions are of critical importance for planning the hardware configuration. For example, the temperature range in the application area is one of the limiting factors for selecting possible devices. Fail-safe capability could be another requirement, for example.

The TIA Selection Tool (select Automation technology  TIA Selection Tool and follow the instructions) offers you support.

Notes

TIA Selection Tool requires Java.

To obtain the device specifications online, look for the manual described as "Product Manual" or "Manual" among the various manuals listed.

### TIA Portal – Project view and portal view

The TIA Portal has two important views. When started, the TIA Portal displays the portal view by default. This view makes the first steps easier, especially for beginners.

The portal view provides you with a task-oriented view of the tools for working with the project. Here, you can quickly decide what you want to do and open the tool for the task in hand. When needed, the system switches to the project view automatically for a selected task.

Figure 2 shows the portal view. You have the possibility to change between this view and the project view at the bottom left.

Figure 2: Portal view

The project view, as shown in Figure 3, is used for the hardware configuration, programming, creation of the visualization and many other tasks.

With the default setting, the menu bar containing the toolbars is at the top, the project tree containing all the components of a project is on the left and the so-called task cards containing instructions, libraries, etc. are on the right.

If an element is selected in the project tree (for example, the device configuration), it is displayed in the middle and can be worked on there.

Figure 3: Project view

### Basic settings for the TIA Portal

The user can make customized default settings for certain settings in the TIA Portal. A couple of important settings are shown here.

In the menu of the Project view, select  "Options" and then  "Settings".

A basic setting is the choice of user interface language and the language for the program display. English is used for both settings in this document.

In the  "General" item of "Settings", select  "English" for "User interface language" and  "International" for "Mnemonics".

Note:

These settings can always be changed back and forth between "English" and "International".

To use safety CPUs (e.g. CPU 1512SP F-1 PN) without the use of safety technology, we recommend you disable automatic generation of the safety program before creating a project.

In the  "STEP 7 Safety" item of "Settings", clear the  "Generate default fail-safe program" check box.

### Setting the IP address on the programming device

In order to program the CPU of a SIMATIC S7-1500 controller from a PC, programming device or laptop, a TCP/IP connection is needed.

For the computer and SIMATIC S7-1500 to communicate with one another via TCP/IP, it is important that the IP addresses of both devices match.

First, it is shown how to set the IP address of a computer with 
Windows 10.

Select the network icon in the taskbar at the bottom  and click on  "Network settings".

In the Network settings window that opens, click  "Ethernet" and then  "Change adapter options".

Select the desired  "LAN connection" you want to use to connect to the controller and click  "Properties".

Select the  "Properties" for  "Internet Protocol Version 4 (TCP/IPv4)".

You can now use the following IP address, for example.  IP address: 192.168.0.99 and enter the following  subnet mask 255.255.255.0. Afterwards, apply the settings. ( "OK")

### Setting the IP address in the CPU

The IP address of a SIMATIC S7-1500 is set as follows.

Double-click the icon for the Totally Integrated Automation Portal.
(® TIA Portal V14)

Select  "Online & Diagnostics" and open the  "Project view".

In the project tree, select the previously set network adapter under  "Online access". When you click  "Update accessible devices", you see the IP address (if already set) or the MAC address (if the IP address has not yet been assigned) of the connected SIMATIC S7-1500. Select  "Online & diagnostics".

Under  "Functions", you will find the  "Assign IP address" item. Enter the following IP address and subnet mask, for example:  IP address: 192.168.0.1  Subnet mask: 255.255.255.0. Click  "Assign IP address", and this new address will be assigned to your SIMATIC S7-1500.

Note:

The IP address of the SIMATIC S7-1500 can also be set via the display on the CPU, if this is enabled in the hardware configuration.

If the assignment of the IP address was not successful, you receive a message in the  "Info" window  "General" tab.

### Formatting a memory card in the CPU

If the IP address could not be assigned, the program data on the CPU must be deleted. This is done in 2 steps:  "Format memory card" and  "Reset to factory settings".

First, select the  "Format memory card" function and click the  "Format" button.

Confirm the prompt "Do you really want to format the memory card?" with  "Yes".

Stop the CPU if necessary. ( "Yes")

### Resetting the CPU to factory settings

Before you can reset the CPU memory, you must wait until the formatting of the CPU is finished. Then you need to select  "Update accessible devices" again and select  "Online & diagnostics" of your CPU. To reset the controller, select the  "Reset to factory settings" function and click  "Reset".

Confirm the prompt "Do you really want to reset the module?" with  "Yes".

Stop the CPU if necessary. ( "Yes")

### Reading out the firmware version of the CPU 1512SP F-1 PN

Before you can read out the firmware version of the CPU 1512SP F-1 PN, you must select  "Update accessible devices" again and select  "Online & diagnostics" of your CPU 1512SP F-1 PN. With the menu command  "Diagnostics"  "General" you can read out the short description, order number, hardware version and firmware version.

# Task

Create a project and configure the following modules of your hardware that correspond to the trainer package SIMATIC CPU 1512SP F-1 PN with software.

1x CPU 1512SP F-1 PN for ET 200SP, central processing unit with 300 KB work memory for program and 1 MB work memory for data, 1 interface, PROFINET IRT with 3-port switch, 48 ns bit performance, memory card (order number: 6ES7512-1SK01-0AB0)

2x DI 8x24VDC/0.5A HF (order number: 6ES7131-6BF00-0CA0)

2x DQ 8x24VDC/0.5A HF (order number: 6ES7132-6BF00-0CA0)

1x server module (order number: 6ES7 193-6PA00-0AA0)

# Planning

Because a new system is involved, a new project must be created.

The hardware is already specified for this project with the SIMATIC CPU 1512SP F-1 PN with software trainer package. For this reason, no selection has to be made. Instead, the listed modules of the trainer package only have to be inserted in the project. In ensure the correct modules are inserted, the order numbers from the task should be checked one more time directly on the mounted devices.

It is customary to start with the CPU and then insert the signal modules in the associated slots. See Table 1.

For the configuration, the Ethernet interface must be set for the CPU and the address ranges of the digital inputs and outputs must be adapted.

| Module | Order number | Slot | Address range |
| --- | --- | --- | --- |
| CPU 1512SP F-1 PN | 6ES7512-1SK01-0AB0 | 1 |  |
| DI 8x24VDC/0.5A HF | 6ES7131-6BF00-0CA0 | 2 | DI 0 |
| DI 8x24VDC/0.5A HF | 6ES7131-6BF00-0CA0 | 3 | DI 1 |
| DQ 8x24VDC/0.5A HF | 6ES7132-6BF00-0CA0 | 4 | DQ 0 |
| DQ 8x24VDC/0.5A HF | 6ES7132-6BF00-0CA0 | 5 | DQ 1 |
| Server module | 6ES7 193-6PA00-0AA0 | 6 |  |

Table 1: Overview of planned configuration

To complete the configuration, the HW configuration must be compiled and downloaded. Configuration errors can be detected during compilation and incorrect modules can be detected when the controller is started (only possible when configured hardware is present and identical). The checked project must be saved.

# Structured step-by-step instructions

You can find instructions on how to perform planning below. If you already have a good understanding of everything, it is sufficient to focus on the numbered steps. Otherwise, simply follow the steps of the instructions illustrated below.

## Creating a new project

Double-click the icon for the Totally Integrated Automation Portal.
(® TIA Portal V14)

In the portal view, select  "Create new project" under the "Start" menu command.

Edit Project name, Path, Author and Comment as appropriate and click  "Set".

The project is created and opened, and the "Start" menu "First steps" opens automatically.

## Inserting the CPU 1512SP F-1 PN

In the  "Start"  portal, "First steps"  select "Devices & networks"  "Configure a device".

In the "Devices & networks" portal, the "Show all devices" menu opens.

Switch to the "Add new device" menu.

The specified model of the CPU will now be added as a new device
(Controllers  SIMATIC ET 200 CPU  ET 200SP CPU  CPU 1512SP F-1 PN  6ES7512-1SK01-0AB0  V2.1)

Assign a device name (Device name  "CPU_1512SP").

Select "Open device view".

Click "Add".

Notes:

A desired CPU may come in multiple variants that differ in their functional scope (work memory, integrated memory, technology functions, etc.). In this case, you should ensure that the selected CPU corresponds to the actual hardware present.

Different firmware versions are often available for the hardware. In this case, it is recommended that the (already preselected) latest firmware be used and that the CPU be upgraded if necessary.

The TIA Portal now automatically changes to the project view and displays the selected CPU 1512SP F-1 PN in slot 1 in the device configuration.

Note:

You can now configure the CPU there according to your specifications. Settings for the PROFINET interface, the startup characteristics, the cycle, the communication load and many other options are possible here.

## Configuration of the Ethernet interface of the CPU 1512SP F-1 PN

Double-click the CPU to select it.

In the  "Properties" tab, open the  "PROFINET interface [X1]" menu and select the  "Ethernet addresses" entry there.

Under "Interface networked with", there is only one entry available: "Not networked".

Add an Ethernet subnet with the  "Add new subnet" button.

Keep the preset "IP address" and "Subnet mask" here.

## Configuration of fail-safe operation of the CPU 1512SP F-1 PN

Change to the  "F-activation" item of the  "Fail-safe" menu and select  "Disable F-activation" there.

Answer the question "Do you want to continue?" with  "Yes".

## Configuration of access level for the CPU 1512SP F-1 PN

Change to the  "Protection & Security" menu and select  "Full access incl. fail-safe (no protection)" for the access level.

Note:

The "Full access incl. fail-safe (no protection)" setting is recommended because a safety program is not created here and we also do not have to assign a password.

## Inserting the digital input modules DI 8x24VDC HF

- Select the correct module in the hardware catalog ( Hardware catalog  Catalog  DI  DI 8x24VDC HF (order number 6ES7131-6BF00-0CA0)  Version: V2.0)

Insert the two digital input modules by dragging them into slots 2 and 3.

Note:

For selection of the digital input module, you can simply enter the order number in the Search field and click the "Search down"  icon. The hardware catalog is opened at the right place.

Note:

When you double-click a module in the Hardware catalog, it is inserted at the next available suitable slot.

## Inserting the digital output modules DQ 8xDC24V / 0.5A HF

Select the correct module from the hardware catalog (Hardware catalog  Catalog  DQ  DQ 8xDC24V/0.5A HF (order number 6ES7132-6BF00-0CA0)  Version: V2.0)

Insert the digital output modules in slots 4 and 5.

Note:

If you have skipped a slot, you must close the gap before compiling to avoid an error message.

## Changing components in the hardware configuration

If it is necessary to change a component in the hardware configuration with a newer version or another type, this can be done by right-clicking the component and selecting "Change device". A dialog opens where you can select a new component for the exchange and confirm the selection with "OK". ( Change device  OK)

Note:

If the new component is not available for selection, it is not compatible with the previous component. In this case, the old component must be deleted first and the new component must then be inserted from the hardware catalog.

## Inserting the server module

Select the correct server module with the correct order number and version from the hardware catalog. Insert the server module in slot 5. ( Hardware catalog  Server module  6ES7 193-6PA00-0AA0  Version: V1.1)

Note:

If you forget to insert the server module, it will be created automatically when the device configuration is compiled.

## Configuration of address ranges for DI/DO: 0…1

The address ranges of the input and output cards must now be checked and, if necessary, adapted. Inputs and outputs (DI/DO) should have an address range of 0…1. ( Device overview  DI  I addresses: 0/1  D0  Q addresses: 0/1)

Note:

To display and hide the device overview, click the small arrows next to "Device data" on the right side of the hardware configuration.

## Configuration of the potential groups of the BaseUnits

To change the potential group of a BaseUnit, select the associated module and open the "Potential group" section in the general properties. (Slot 3 → Properties → General → Potential group)

Select the "Enable new potential group (light BaseUnit)" option.

The BaseUnit in the configuration has now become a light BaseUnit.

Repeat these steps for slots 4 and 5 and compare the device configuration with the figure below.

## Saving and compiling the hardware configuration

Before you compile the configuration, you should save your project by clicking the ®  button. To compile your CPU with the device configuration, first select the  "CPU_1512SP [CPU 1512SP F-1 PN]" folder and click the  "Compile" icon .

Note:

You should use the "Save project" function periodically when working on a project because automatic saving of the project does not occur. The only time you are prompted to save the project is when you close the TIA Portal.

If the configuration is compiled without errors, you see the following:

## Downloading the hardware configuration to the device

To download your complete CPU to the device, select the  "CPU_1512SP [CPU 1512SP F-1 PN]" folder again and click the  "Download to device" icon .

The manager for configuration of connection properties (Extended download) opens.

First, the interface must be correctly selected. This is done in three steps:

Type of PG/PC interface:  PN/IE

PG/PC interface:  here e.g.: Intel(R) Ethernet Connection I217-LM

Note:

The PG/PC interface used here depends on the Ethernet interface adapter installed in the programming device.

Connection to interface/subnet  "PN/IE_1"

The  "Show all compatible devices" field must then be selected and the search for devices in the network must be started by clicking the   button.

If your CPU is displayed in the "Compatible devices in target subnet" list, select it and start the download. ( CPU 1512SP F-1 PN  "Load")

You first receive a preview. Continue with  "Load".

Note:

In the "Load preview", you should see the  icon in each line in which actions will be performed. You can see additional information in the "Message" column.

Select the  "Start all" option before completing the download operation with  "Finish".

The project view is opened again automatically after a successful download operation. A download report appears in the "General" tab of the Info field. This can be helpful for troubleshooting if the download operation was unsuccessful.

## Downloading the hardware configuration to the PLCSIM simulation (optional)

If hardware is not available, the hardware configuration can be loaded alternatively to a PLC simulation (PLCSIM).

First, start the simulation by selecting the  "CPU_1512SP [CPU 1512SP F-1 PN]" folder and clicking the   "Start simulation" button.

Confirm the notice "Starting simulation will disable all other online interfaces" with  "OK".

The "PLCSIM" software is started in a separate window in the compact view.

The manager for configuration of connection properties (Extended download) opens shortly thereafter.

The interface must be correctly selected to start. This is done in three steps:

Type of PG/PC interface:  PN/IE

PG/PC interface:  PLCSIM

Connection to interface/subnet  "PN/IE_1"

The  "Show all compatible devices" field must then be selected and the search for devices in the network must be started by clicking the   button.

If your panel is displayed in the "Compatible devices in target subnet" list, it must be selected before the download can be started. ( "CPU-1500 Simulation"  "Load")

You first receive a preview. Continue with  "Load".

Note:

In the "Load preview", you should see the  icon in each line in which actions will be performed. You can see additional information in the "Message" column.

The  "Start all" option is now selected before the download operation can be completed with  "Finish".

The project view is opened again automatically after a successful download operation. A download report appears in the "General" tab of the Info field. This can be helpful for troubleshooting if the download operation was unsuccessful.

The compact view of the PLCSIM simulation is as follows. However, no simulation project has been created yet. If you click the   icon, you can now change to the project view.

The PLCSIM simulation is as follows in the project view. Here, you must first click the   icon to  "Create" a "New project".

If you double-click the  "Device configuration", you can view the downloaded configuration in the project view. You can also set input signals and monitor output signals later for testing the programs. You can change back to the compact view of the simulation by clicking the   icon in the menu bar.

Note:

You cannot detect hardware configuration errors because a hardware simulation is involved.

## Archiving the project

To archive the project, select the  "Archive" command in the  "Project" menu.

Confirm the prompt "Save project?" with "Yes".

Select a folder in which you want to archive your project and save the project as file type "TIA Portal project archive". ( "TIA Portal project archive"  "SCE_EN_012-110 Hardware Configuration S7-1500 CPU 1512SP F-1 PN …"  "Save").

## Checklist – step-by-step instructions

The following checklist helps trainees/students to independently check whether all steps of the step-by-step instruction have been carefully completed and enables them to successfully complete the module on their own.

| No. | Description | Completed |
| --- | --- | --- |
| 1 | Project created. |  |
| 2 | Slot 1: CPU 1512SP F-1 PN with correct order number |  |
| 3 | Slot 1: CPU 1512SP F-1 PN with correct firmware version |  |
| 4 | Slots 2…3: Digital input modules with the correct order number |  |
| 5 | Slots 2…3: Digital input modules with the correct firmware version |  |
| 6 | Slots 2…3: Address ranges of the digital input modules correct |  |
| 7 | Slots 4…5: Digital output modules with the correct order number |  |
| 8 | Slots 4…5: Digital output modules with the correct firmware version |  |
| 9 | Slots 4…5: Address ranges of the digital output modules correct |  |
| 10 | Slot 6: Server module |  |
| 11 | Modules have set all the correct potential groups for the BaseUnits |  |
| 12 | Hardware configuration compiled without error message |  |
| 13 | Hardware configuration downloaded without error message |  |
| 14 | Project successfully archived |  |

# Exercise

## Task description – Exercise

The hardware configuration is to be expanded to include the trainer package SIMATIC ET 200SP Analog Modules Expansion. Insert the following missing modules. Select slots 6 and 7 for the analog input modules and slot 8 for the analog output module. The server module is moved to slot 9. Set the address range starting from 64 for the analog modules. Light BaseUnits will be used here.

2X AI 2XU/I 2-/4-WIRE HS (order number: 6ES7134-6HB00-0DA1)

1X AQ 2XU/I HS (order number: 6ES7135-6HB00-0DA1)

| Module | Order number | Slot | Address range |
| --- | --- | --- | --- |
| AI 2xU/I 2-/4-wire HS | 6ES7134-6HB00-0DA1 | 6 | 64…67 |
| AI 2xU/I 2-/4-wire HS | 6ES7134-6HB00-0DA1 | 7 | 68…71 |
| AQ 2xU/I HS | 6ES7135-6HB00-0DA1 | 8 | 64…67 |

Table 1: Analog modules of the CPU 1512SP F-1 PN

## Planning

Plan the implementation of the task by yourself.

## Checklist – Exercise

The following checklist helps trainees/students to independently check whether all steps of the exercise have been carefully completed and enables them to successfully complete the module on their own.

| No. | Description | Completed |
| --- | --- | --- |
| 1 | Slots 6…7: Analog input modules with the correct order number |  |
| 2 | Slots 6…7: Analog input modules with the correct firmware version |  |
| 3 | Slots 6…7: Address ranges of the analog input modules correct |  |
| 4 | Slot 8: Analog output module with the correct order number |  |
| 5 | Slot 8: Analog output module with the correct firmware version |  |
| 6 | Slot 8: Address range of the analog output module correct |  |
| 7 | Slot 9: Server module |  |
| 8 | Modules have set all the correct potential groups for the BaseUnits |  |
| 9 | Hardware configuration compiled without error message |  |
| 10 | Hardware configuration downloaded without error message |  |
| 11 | Project successfully archived |  |

# Additional information

- More information for further practice and consolidation is available as orientation, for example: Getting Started, videos, tutorials, apps, manuals, programming guidelines and trial software / firmware, under the following link:

siemens.com/sce/s7-1500

Preview "Additional information"

Further Information

Siemens Automation Cooperates with Education
siemens.com/sce

SCE Learn-/Training Documents
siemens.com/sce/documents

SCE Trainer Packages
siemens.com/sce/tp

SCE Contact Partners 
siemens.com/sce/contact

Digital Enterprise
siemens.com/digital-enterprise

Industrie 4.0 
siemens.com/future-of-manufacturing

Totally Integrated Automation (TIA)
siemens.com/tia

TIA Portal
siemens.com/tia-portal

SIMATIC Controller
siemens.com/controller

SIMATIC Technical Documentation 
siemens.com/simatic-docu

Industry Online Support
support.industry.siemens.com

Product catalogue and online ordering system Industry Mall 
mall.industry.siemens.com

Siemens 
Digital Industries 
P.O. Box 4848
90026 Nuremberg
Germany

Subject to change and errors
© Siemens 2019

siemens.com/sce